# Introduction

The WPI Husky Technology fex21 Application Programs Interface (API) library is a set of functions and structures packaged in a Dynamic Linked Library (DLL) which is shipped on the fex21 as part of the Windows CE ® operating system.  The DLL provides access to the low-level functions of the machine allowing the software developer a greater level of control over facilities such as battery management and display control than is provided by the standard win32 API libraries.

The API is important for those developers who wish to make use of the fex21’s unique features from a familiar programming environment such as Microsoft Visual Studio ® 6.0 for C/C++ ® 6.0.

### About This Document

This document provides a guide to the fex21API DLL provided with the Husky fex21.

This document assumes that the user is already familiar with their development environment and has at least a general understanding of the Windows CE ® operating system and using DLLs in their chosen programming language.

This book does not cover any general Graphical User Interface (GUI) programming or other Windows functionality, documentation of which can be found with the development environment chosen.

This Book is in three parts.  Part 1, “Overview,” introduces the library, describes the general rules for its use and summarises the main categories of routines.  Part 1 contains the following chapters:

- Chapter 1, “Using fex21API.DLL,” gives general rules for using the library and any special considerations.

- Chapter 2, “Functions by Category,” lists the functions by category and discusses any considerations that apply to each category.  This chapter makes it easy to locate the required functions by task, detailed descriptions can then be found in Part 2.

- Chapter 3, “Types, Structures and Constants,” describes the abstract data types used within the library.

Part 2, “Function Descriptions,” provides detailed descriptions of the functions in alphabetical order.

Part 3, “The ‘Util’ Control Panel Applets,” describes how to use the GUI functions of the fex21Utl module.

### Further Reading

The following books cover a variety of topics, which may be useful when designing and implementing applications for fex21.  With the exception of its own user documentation, WPI Husky Technology does not endorse these books or recommend them over others on the same subject.

- WPI Husky Technology.  Husky fex21 User Guide.  WPI Husky Technology, 1998.		General information on the fex21 computer.

- Microsoft.  Microsoft® Windows® CE Programmer’s Guide. Microsoft Press, 1998.		A guide to writing applications for Microsoft® Windows® CE based platforms.

### Examples

All the major examples in this book assume the user is developing using Microsoft Visual C/C++ 6.0.

### Document Conventions

This book uses the following typographic conventions:

| Example | Description |
| --- | --- |
| FEX21API.H | Uppercase letters indicate filenames and terms used at the operating system command level. |
| #### int, __cplusplus | Bold type indicates reserved words. |
| #### Expression | Italic type is used to indicate information placeholders which should be supplied by the programmer. |
| #### [[option]] | Items in double square brackets are optional. |
| #### #pragma pack { 1 ¦ 2 } | Braces and a vertical bar indicate a choice among two or more items, one and only one of which must be selected (unless the braces are surrounded by double square brackets as above). |
| #### void myfunc(void); | This font is used for examples. |
| #### Operating System (OS) | The first time an acronym is used, it is spelled out in full. |

### Trademarks

Windows ®, Microsoft ®, Visual C/C++ ® and Visual Studio are all registered trademarks of Microsoft Corporation.

All other trademarks are acknowledged.

### About Husky

We can be contacted by any of the following means:

Telephone:	+44 (0) 24 7660 4040	(before 22nd April 2000: +44 1203 604040)

Facsimile:	+44 (0) 24 7660 3060	(before 22nd April 2000: +44 1203 603060)

Email:		support@husky.co.uk

Address:	WPI Husky Technology,

Eden Road,

Walsgrave Triangle Business Park,

Coventry	CV2 2TB

England

Overview	Part 1

#### Chapter	1 Using fex21API.DLL

2 Functions by Category

3 Types, Structures and Constants

4 Dynamic Link Library Functions

The first part of this book introduces the fex21API.DLL library, describes the general rules for its use and summarises the main categories of routines.

Chapter 1 gives general rules for using the library and any special considerations.

Chapter 2 lists the functions by category and discusses any considerations that apply to each category.

Chapter 3 describes the abstract data types used within the library.

Chapter 4 provides a detailed description of each of the functions listed in Chapter 2.

Using fex21API.DLL	Chapter 1

This chapter provides basic information on how to use the library routines from Visual C/C++.

### Application requirements for Visual C/C++

The simplest way of accessing the fex21API.dll from a C/C++ application will require the fex21API.h file to be included in every C or CPP module, which references any of the library functions, structures or constants.

This is done using the standard #include directive:

#include <fex21API.h>

The fex21API.h file should be placed in one of the directories on the includes search path.

In addition, the linker will require the stub functions from the fex21API.lib file in order to be able to access the DLL at run-time.  This file will need to be included in your project settings along with its path.

For more information about installing and using static linked libraries see your development environment documentation.

An alternative method is to load the library when it is required using the LoadLibrary() function and retrieving the addresses for only those functions which are required using the standard GetProcAddress() function, then closing the dll once it is finished.

For example, to read the LCD type the following code may be used:

typedef BOOL (*PTYPEFUNC)(BYTE *, BYTE *, BYTE *);

char GetLCDType( void )

{

BYTE bType = ‘\0’;

// Load the Fex21API utilities library

HINSTANCE hDllInst = LoadLibrary( TEXT("Fex21API.dll"));

if( hDllInst != NULL )

{

BOOL (*keybGetID)(BYTE *, BYTE *, BYTE *) =

(PTYPEFUNC)GetProcAddress( hDllInst, TEXT("keybGetID"));

keybGetID( NULL, NULL, &bType );

FreeLibrary( hDllInst );

}

return (char)bType;

}

### Using the DLL from Visual C/C++

All of the functions declared in fex21API.h may be called from any module in which it is included, in the same way as any statically linked function.

For example, a function to read the contrast level a code fragment could be as follows:

#include <fex21API.h>

int GetContrast( void )

{

BYTE bContrast;

if( !lcdGetContrast( &bContrast ))

return –1;

else

return bContrast;

}

Bit fields are provided to access the elements of the complex data structures allowing you to refer to bit fields in the structure as though they were individual data members.  This is a standard C/C++ construction whose details can be found in your compiler documentation.

For example: the structure ISISBATTERYSTATUS is defined as follows:

typedef struct tagISISBATTERYSTATUS

{

WORD StdByStat      : 2;  /* Standby battery status */

WORD BatChgd        : 1;  /* Battery changed flag */

WORD BatFault       : 1;  /* Smart battery fault detected flag */

WORD BatType        : 1;  /* Battery type 1=Smart, 0=Not smart */

WORD BatStat        : 3;  /* Smart battery status */

WORD BatLife        : 7;  /* % remaining or 127 if unknown */

WORD ChrgConn       : 1;  /* Charger connected flag */

WORD CapLeft        : 14; /* Estimated mAh */

WORD CapUnreliable  : 1;  /* CapLeft unreliable */

WORD ChrgOk         : 1;  /* Allowed if smart battery */

} ISISBATTERYSTATUS, *PISISBATTERYSTATUS;

To access the BatLife member of the structure which occurs in bits 0..6 of BYTE 2 as a WORD variable you would use the following:

WORD GetPercentRemaining( void )

{

ISISBATTERYSTATUS sBatStat;

If(( !psuGetBatteryStatus( &sBatStat ))||

( sBatStat.BatLife > 100 ))

Return (WORD)–1;

Else

Return sBatStat.BatLife;

}

Note: No masking or shifting is required in C/C++ because the structure member was declared as a bit field.  sBatStat.BatLife’s range will be 0 to 100 or 127 if an error occurred.

Functions by Category	Chapter 2

This chapter provides a list of the functions provided by the DLL grouped by category.  Further information on each function can be found in Part 2.

The main categories of routines are

- General DLL information

- Keyboard manipulation

- Power status

- Display

- Non-volatile RAM (NVRAM)

- Back-up Real Time Clock (RTC)

- Communications ports

### General DLL information

The general DLL information routines provide the application with information about the DLL, which may affect the interface.

| #### Routine | Description |
| --- | --- |
| dllGetVersion | Retrieves the version and build date of the fex21API.DLL file. |

### Keyboard Manipulation

The keyboard manipulation routines provide low-level access to the micro controller keyboard hardware.

There are very few instances when an application will need to use any of these functions since all keyboard input is handled by the keyboard device driver and the Windows CE kernel.

The only one that may be safely used by an application is described below.

| #### Routine | Description |
| --- | --- |
| keybGetID | Reads the micro-controller code version and LCD type prefix |
| keybGetDefault | Reads the current keyboard layout index. |
| keybSetDefault | Changes the current keyboard layout index. |

### Power Status

The power status functions allow access to the battery status and information and allow some control over charging routines.

| #### Routine | Description |
| --- | --- |
| psuGetBatterySerialNo | Retrieves the serial number of the main battery. |
| psuGetBatteryStatus | Retrieves charge status information. |
| psuGetBatteryInfo | Retrieves information about the main battery. |
| psuGetChargingEnabled | Indicates whether main battery charging is enabled. |
| psuEnableCharging | Enables charging for rechargable batteries |
| psuDisableCharging | Disables charging for rechargable batteries |
| psuGetAllowSuspend | Reads the flag indicating whether the unit can turn off whilst the battery is charging. |
| psuSetAllowSuspend | Changes the flag indicating whether the unit can turn off whilst the battery is charging |

### Display

The display routines allow the developer to change and review the settings of the contrast level and backlight.

| #### Routine | Description |
| --- | --- |
| lcdGetBacklight | Gets the current backlight level and display information. |
| lcdSetBacklight | Changes the current backlight level and timeout settings. |
| lcdGetContrast | Gets the current contrast level |
| lcdSetContrast | Changes the current contrast level |

These functions will have subtly different effects depending on which display type is on the unit, therefore the developer must be sure to check the display type before changing any of the other settings.

### Non-volatile RAM (NVRAM)

The non-volatile RAM routines allow the user to store and retrieve information over complete power loss events.  Information stored in the micro controller NVRAM is normally unit specific information such as identification strings and passwords which should not have generic default values.

| #### Routine | Description |
| --- | --- |
| nvramReadBlock | Reads one 16 byte block from NVRAM |
| nvramWriteBlock | Writes one 16 byte block to NVRAM |
| nvramGetSerialNumber | Reads the unit’s unique serial number from NVRAM as a UNICODE string. |
| nvramGetUUID | Reads the unit’s Universally Unique IDentifier (UUID) as binary data |
| nvramGetUUIDStr | Reads the unit’s Universally Unique IDentifier (UUID) as a human readable string. |

Note that the unit serial number must never be changed since doing so may result in some devices which rely on unique machine Id’s failing to work correctly.

### Back-up Real Time Clock (RTC)

The back-up RTC routines allow the user to check and change the back-up RTC, kept by the micro controller.  This RTC is essential for correct battery maintenance.

| #### Routine | Description |
| --- | --- |
| rtcGetBackup | Reads the current value held in the back-up RTC |
| rtcSetBackup | Set the value held in the back-up RTC |

The back-up RTC uses a counter rather than discrete seconds, minutes, etc registers and therefore will not be effected by year roll over problems.

### Communications ports

The communications ports routines allow the developer to redirect serial output between the main RS232 Dtype connector and the Communications and Charging cradle studs, or IR connector depending on which port is being configured.

They also allow the application to control the power output on the 9pin Dtype connectors.

| #### Routine | Description |
| --- | --- |
| comSetPort1Connection | Redirect serial output between RS232 connector and charging cradle studs. |
| comGetPort1Connection | Checks the current connector selection. |
| comSetPort2Connection | Redirect serial output between the secondary RS232 connector and IR port. This function can only be used when port2 is not already open. |
| comGetPort2Connection | Checks the current connector selection. |
| comSetPort1PowerOut comSetPort2PowerOut | Assert power output on RS232 Dtype connectors. |
| comGetPort1PowerOut comGetPort2PowerOut | Checks if power output on RS232 Dtype connectors is asserted. |

Types, Structures and Constants	Chapter 3

This chapter describes the types, structures and constants used by the DLL to exchange information with the application.

The structures are described in a general byte by byte, bit by bit.

### Types

The only new types used by the DLL are provided for convenience only and are used to represent the structures defined below.

The C/C++ interface uses some of the convenience integer types (BYTE, WORD, etc) declared in WINDOWS.H

Any strings returned by the DLL are in C/C++ NULL terminated format and consist of UNICODE characters unless it is stated differently in their description.

### Structures

All data is passed to the DLL as arrays of Bytes.  The structures defined in the interface modules are provided to allow simplified access to the individual bit field members of the Byte elements of the arrays.

The tables have the following format

| #### Byte | Bits | ##### R/w | Value & Range | Description |
| --- | --- | --- | --- | --- |

Where:

| #### Byte | The byte of the structure to which the field belongs. This field is not particularly relevant to C/C++ since this information is handled by the structure itself. |
| --- | --- |
| Bits | The bits within the byte that the field occupies. This field is not particularly relevant to C/C++ since this information is handled by the structure’s bitfields. |
| R/w | Shows whether the field is read only (R), write only (W) or both (R/W). Read only bits should not be changed since this may result in unpredictable behaviour. Write only bits may not contain meaningful or consistent information when read. |
| Value & Range | Shows either the values the field may contain and their meanings where relevant, or the range of values which the field may take. |
| Description | Describes the meaning of the field, its relationship to any other fields and when the field is and is not applicable. |

#### ISISBATTERYSTATUS

This structure is used to retrieve the current status of the fex21’s batteries.

It is organised as a block of 4 bytes with the bit fields defined as follows.

| #### Byte | Bits | ##### R/w | Value & Range | Description |
| --- | --- | --- | --- | --- |
| Byte 1 | Bit 7-5 | R | 000 = Not Known | Main Battery Status |
|  |  |  | 001 = Critical |  |
|  |  |  | 010 = Low |  |
|  |  |  | 011 = High |  |
|  |  |  | 100 = Charging |  |
|  |  |  | 101 = N/K (wakeup charge) |  |
|  |  |  | 110 = Reserved |  |
|  |  |  | 111 = No Battery |  |
|  | Bit 4 | R | 1 = Smart Battery | Alkaline or Smart Battery. (Valid only if bits 7 – 5 = 001-100 inc.) |
|  |  |  | 0 = Alkaline |  |
|  | Bit 3 | R | 1 = Smart Battery Fault | A part of smart battery circuit is faulty (valid only if bit 4 = smart battery) Detailed information of the fault can be obtained by using psuGetBatteryInfo(). |
|  | Bit 2 | R | 1 = Battery Changed | Main battery being changed or has been changed (bit will then stay set for first successful host read, then clear) |
|  | Bit 1,0 | R | 00 = Not Known | Standby Battery Status (Will read always ‘Not Known’ since the standby battery is always on charge while the unit is powered on). |
|  |  |  | 01 = Critical |  |
|  |  |  | 10 = Low |  |
|  |  |  | 11 = High |  |
| Byte 2 | Bit 7 | R | 1 = AC line powered | Indicates that the external power supply is connected and being used to power the unit. |
|  | Bit 6-0 | R | 0 – 100 dec. or if not known returns 127 dec | Percentage battery life remaining. |
| Byte 3 | Bit 7-0 | R | (LSB) 0 – 16,383 (dec.) (MSB) | Estimated remaining capacity in mAh If not known returns 0x3FFF (16383 dec.) |
| Byte 4 | Bit 5-0 | R |  |  |
|  | Bit 6 | R | 1- Estimate unreliable | Percentage life , and remaining capacity estimates are unreliable, due to the time since the battery was last fully charged |
|  | Bit 7 | R | 1 = Charging allowed | Main battery is ok to charge |

If bit 3 of byte 1 indicates that there is a fault in the main battery, psuGetBatteryInfo() should be used to determine the exact nature of the fault.

If bit 4 of byte 1 indicates that no smart battery is fitted the rest of the structure except bit 7 of byte 2 (AC line connected), will contain meaningless values.

#### ISISBATTERYINFO

This structure is used to retrieve detailed information on the fex21’s main smart batteries if fitted.  The function psuGetBatteryStatus() should be called to determine whether smart batteries are fitted to determine whether this structure contains valid information.

It is organised as a block of 9 bytes with the bit fields defined as follows.

| #### Byte | Bits | ##### R/w | Value & Range | Description |
| --- | --- | --- | --- | --- |
| Byte 1 | Bit 7 | R | 1 = Smart Battery fitted | 0 = No smart battery - Ignore all other fields. |
|  | Bit 6 | R | 1 = Smart Battery OK | This bit is a logical AND of bits 5 – 2 below |
|  | Bit 5 | R | 1 = Gas Gauge OK | 0 = Gas gauge is not answering, or giving wrong data |
|  | Bit 4 | R | 1 = E2Prom Answering + ID CRC OK | 0 = E2prom is not answering / corrupt - Ignore bytes 2 - 9) |
|  | Bit 3 | R | 1 = Temperature In Charging Range | 0 = Temperature Out Of Charging Range (Charging will not occur until temperature is in range) |
|  | Bit 2 | R | 1 = Thermistor OK | 0 = Thermistor out of valid range i.e. faulty |
|  | Bit 1 | R | 0 = Under range 1 = Over range | Bit valid if Bit 2 or 3 is not set Indicates direction of range violation. |
|  | Bit 0 | R | 1 = Charging OK | 1 = Fault While Charging Indicates either over voltage or over temperature while charging. |
| Byte 2 | Bit 7–0 | R | Serial number (unsigned byte) 0 – 255 | Daily Serial number |
| Byte 3 | Bit 4–0 | R | Day of Month 1–31 or 0 | Day of month of manufacture (0 = Date of manufacture not known) |
| Byte 4 | Bit 7, 6 | R | Chemistry | 00 – NiCad, 01 = NiMH, 10 – LiIon |
|  | Bit 5, 4 | R | Major Battery Rev Indicator | 00 = current batteries, 01 – 11 = future use |
|  | Bit 3–0 | R | Month 1 – 12 | Month of manufacture |
| Byte 5 | Bit 7–0 | R | Year 99 – 255 | Year of manufacture - Offset from 1900 |
| Byte 6 | Bit 7–0 | R | (LSB) Battery Capacity (MSB) (0 – 65535 mAh) | Manufacturer quoted capacity |
| Byte 7 | Bit 7–0 |  |  |  |
| Byte 8 | Bit 7–0 | R | (LSB) Battery Charge Cycles (MSB) (1 – 8191) | 0 = Not Known 8191 charge cycles > 8190 |
| Byte 9 | Bit 4–0 |  |  |  |
|  | Bit 7 | R | 1 = E2Prom PARA CRC OK & has always been OK | 0 = E2Prom could be corrupt. (No confidence should be placed on Battery Charge Cycles Value - Note that battery charging will still be allowed). |

Bits 0 to 3 of byte 1 are used to indicate the reason for any failure in charging.

A 0 value for bit 0 indicates that charging has been aborted for some reason.  In this case charging will not recommence until the battery has been removed and replaced and the unit soft reset (see User Guide for details).

Bit 1 is used to indicate the direction in which a range violation has occurred.  It is only valid if either bit 2 or bit 3 is clear, indicating that a range violation has occurred.

Bit 2 is 0 if an electrical fault has caused the battery thermistor voltage to go out of range (bit 1 will indicate which direction).  If this bit is clear the contents of bit 3 should be ignored since the thermistor is not functioning properly to give a correct temperature reading.

Bit 3 is 0 if the main battery temperature has gone out of the safe charging range (bit 1 will indicate which direction).  Charging will resume once the battery temperature has returned to a suitable level.

#### Battery serial numbers

The unique battery identifier is constructed from the daily serial number with manufacture date.

I.e.	189 23 10 99

Serial number

Day Month Year of manufacture

If more than 256 batteries are manufactured on one day, the date will be rolled over, and the serial number will start again. The date will then be allowed to catch up, unless the serial number is required to roll over again.

The battery serial number as printed on the battery itself is constructed as a 3-byte block, displayed as 6 hexadecimal characters.  The 3-byte block is constructed as follows.

| #### Byte | Bits | ISISBATTERYINFO data source | Description |
| --- | --- | --- | --- |
| Byte 1 | Bit 7-6 | Revision | Battery board build revision |
|  | Bit 1-5 | YearOfMan - 99 | Year field offset from 1999 (range 0-31) |
|  | Bit 0 | MonOfMan (MSB) | MSB of month of manufacture field |
| Byte 2 | Bit 7-5 | MonOfMan (3LSBs) | 3 LSBs of month of manufacture field |
|  | Bit 4-0 | DayOfMonth | Day of month of manufacture |
| Byte 3 | Bit 7-0 | SerNo | Serial number field |

#### ISISBACKLIGHT

This structure is used to get and set the backlight level and timeout configuration.

On transflexive (TR) colour and monochrome units the backlight can be turned off completely.  On super twist nematic (STN) colour units the backlight is merely dimmed to a very low level.

Consequently when the backlight timeout expires and the backlight is ‘turned off’, on STN units this results in the backlight being dimmed and on TR and monochrome units the backlight is fully powered off.

It is organised as a block of 2 bytes with the bit fields defined as follows.

| #### Byte | Bits | ##### R/w | Value & Range | Description |
| --- | --- | --- | --- | --- |
| Byte 1 | Bit 7 | W | 1 = Alter Backlight TO 0 = Ignore this byte | When reading the Backlight ignore this bit. When changing the level only this bit should be 0. When changing the timeout configuration this bit should be 1. |
|  | Bit 5,4 | R/W | 00 = Leave this setting 01 = T.O fixed 10 = T.O restart on key 11 = Ditto, resume on key after T.O end | Backlight Timeout mode. When setting the timeout duration only this field should be 0. When setting, non-zero values change the timeout mode (see below). |
|  | Bit 3–0 | R/W | Backlight Time Out value 1 – 15 or 0 | Backlight timeout value in multiples of 30 seconds. I.e. gives 30 secs – 7.5 mins. (0 for no backlight time out) |
| Byte 2 | Bit 7 | W | 0 = Do not alter backlight state 1 = Alter backlight state (see bit 6) | When changing the backlight state this bit must be 1, otherwise it must be 0. |
|  | Bit 6 | R/W | 0 = Backlight Off 1 = Backlight On | Set off and Set on commands are for Mono and TR colour units, on STN colour units these commands will take it to minimum and maximum respectively. |
|  | Bit 5–4 | R | 00 = Mono Display 10 = STN Colour 11 = TR Colour 01 = Reserved | Display Type |
|  | Bit 3 | W | 0 = Do Not Alter Brightness 1 = Alter Brightness (see bits 1 and 2) | When changing the brightness level this bit must be 1, otherwise it must be 0. |
|  | Bit 2, 1 | R/W | Brightness level 0 = dim - 3 = bright | For STN and T/R colour units only. Ignored on monochrome displays. |

#### ISISVERSION

This structure is used to retrieve the DLL version and build information.

Bytes 5 to 36 contain a NULL terminated, 16 character, Unicode (16 bit) string representation of the date on which the system was built in the format Mmm dd yyyy.

E.g.: Aug 26 1999

| #### Byte | ##### R/w | Value & Range | Description |
| --- | --- | --- | --- |
| Byte 1 | W | 0 to FFh Hexadecimal | DLL release number |
| Byte 2 | W | 0 to FFh Hexadecimal | DLL major version number |
| Byte 3 | W | 0 to FFh Hexadecimal | DLL minor version number |
| Byte 4 | W | 0 to FFh Hexadecimal | Reserved |
| Byte 5..36 | W | NULL terminated, 16 character, Unicode string. | DLL build date (Mmm dd yyyy) |
| Byte 37 | W | 0 to FFh Hexadecimal | For future use |
| Byte 38 | W | 0 to FFh Hexadecimal | For future use |

#### ISISRTC

This structure is used to set and read the value of the micro-controller's backup Real Time Clock (RTC).

If this is not set once the values returned by the battery % and chanrge remaining functions may be inaccurate due to the backup RTC being used to determine self-discharge rates.

| #### Byte | Bits | ##### R/w | Value & Range | Description |
| --- | --- | --- | --- | --- |
| Byte 1 | Bit 7 | R | 1 = RTC battery fault | RTC battery has been flat at some time |
|  | Bit 6 | R | 1 = RTC fault | Some unknown fault has occurred |
|  | Bits 5-0 | R/W | 0 to 59 | Seconds value |
| Byte 2 | Bits 7,6 | R | Ignore | Future use |
|  | Bits 5-0 | R/W | 0 to 59 | Minutes value |
| Byte 3 | Bits 7-5 | R | Ignore | Future use |
|  | Bits 4-0 | R/W | 0 to 23 | Hours value (24 hour) |
| Byte 4 | Bits 7-5 | R | Ignore | Future use |
|  | Bits 4-0 | R/W | 1 to 31 | Day of Month |
| Byte 5 | Bits 7-4 | R | Ignore | Future use |
|  | Bits 3-0 | R/W | 1 to 12 | Month |
| Byte 6 | Bits 7-0 | R/W | 99 to 130 | Year offset from 1900 |

Dynamic Link Library functions	Chapter 4

This chapter describes each of the functions provided by the DLL in detail, expanding on the parameters passed, return types and any other considerations of each function.

Each function’s entry will have the following sections:

Description	What the function does, its calling structure, parameters and return types.

Remarks	Further, detailed information on the operation of the function.

Return Value	What the calling process can expect to receive when the function returns.

Example	Some functions will include a short piece of C/C++ code to demonstrate.

### dllGetVersion

#### Description

Retrieves a structure containing the version information for the fex21API.DLL file.

ISISAPI BOOL APIENTRY dllGetVersion( PISISVERSION pDllVer );

###### pDllVer		Points to an ISISVERSION structure to receive the version information.

#### Remarks

This function will fill the ISISVERSION structure passed with the major, minor and release numbers and build date of the loaded version of the DLL.  It is provided for debugging purposes so that applications can distinguish earlier and later versions.

#### Return Value

If successful, the function returns TRUE.  Otherwise it returns FALSE and the content of the structure will be unchanged.

#### Example

void ShowDllVersion( void )

{

ISISVERSION ivDllVer;

TCHAR szText[64];

if( dllGetVersion( &ivDllVer ))

{

wsprintf( szText,

TEXT("Version: %02.2d.%02.2d.%02.2d\r\nBuilt: %s"),

(int)ivDllVer.ReleaseNo,

(int)ivDllVer.MajorVer,

(int)ivDllVer.MinorVer,

ivDllVer.BuildDate );

MessageBox(NULL,szText,TEXT(“Fex21API.Dll version”),MB_OK);

}

}

### keybGetID

#### Description

Retrieves the version information of the microcontroller firmware.

ISISAPI BOOL APIENTRY keybGetID(BYTE *pbRevision, BYTE *pbBuildNo, BYTE *pbDispTyp);

pbRevision	Points to a byte to hold the revision number (hexadecimal).

pbBuildNo	Points to a byte to hold the build number (hexadecimal).

PbDispTyp	Points to a byte to hold the display type (ANSI character).

M	Monochrome micro-controller code fitted.

S	STN (colour) micro-controller code fitted.

T	T/R (colour) micro-controller code fitted.

#### Remarks

This function will fill the bytes with the revision, build and display types for the micro-controller.  If any of the parameters are not required they should be set to NULL.

The pbRevision and pbBuildNo parameters will be filled with a hexadecimal number, while the PbDispTyp parameter will have an ANSI (8 bit) character entered depending on the LCD type.

#### Return Value

If successful, the function returns TRUE.  Otherwise it returns FALSE and the content of the parameters will be unchanged.

#### Example

void ShowLCDType( void )

{

unsigned char cType;

// Only want to get the LCD type so set the others as NULL

if( keybGetID( NULL, NULL, & cType ))

{

switch( cType )

{

case ‘M’:

MessageBox(NULL,TEXT(“Monochrome”),TEXT(“LCD Type”),MB_OK);

break;

case ‘S’:

MessageBox(NULL,TEXT(“STN colour”),TEXT(“LCD Type”),MB_OK);

break;

case ‘T’:

MessageBox(NULL,TEXT(“T/R colour”),TEXT(“LCD Type”),MB_OK);

break;

}

}

}

### keybGetDefault, keybSetDefault

#### Description

These functions control the current default keyboard layout, held in non-volatile memory.  This value is used by the keyboard driver to determine which layout to use to process keypresses to Virtual Key codes (Vkeys) and ultimately, Unicode characters.

ISISAPI BOOL APIENTRY keybGetDefault (DWORD *pdwIdx);

ISISAPI BOOL APIENTRY keybSetDefault (DWORD dwIdx);

pdwIdx		Points to a DWORD to hold the current layout index.

dwIdx		DWORD value of the new layout index.

#### Remarks

When the default layout index is changed the new value is not used until the unit is reset (see User Guide for more information on warm resetting the fex21).

The values used for dwIdx and returned in pdwIdx may be any one of the following keyboard layouts:

- US and UK

- German

- French

- Spanish

- Swedish and Finnish

- Danish and Norwegian

The default value will not be used if a layout file has been explicitly specified in the registry.  Layout files may only be used with custom keyboards and the explicit permission of WPI Husky Technology Ltd.

#### Return Value

If successful, the function returns TRUE.  Otherwise it returns FALSE and system will remain unchanged.

### psuGetBatterySerialNo

#### Description

Retrieves the main rechargeable smart battery serial number (if one is fitted) as a wide character (Unicode) string.

ISISAPI BOOL APIENTRY psuGetBatterySerialNo(TCHAR *szStr);

szStr	Points to a wide character string to hold the serial number

#### Remarks

This function will fill the string with the serial number of the battery if one is fitted.  This should be identical to the number printed on the battery itself.

The string argument should contain enough space for 9 characters plus the terminating NULL

The battery serial number can also be constructed from the ISISBATTERYINFO structure returned by the psuGetBatteryInfo() function described below.  (See chapter 3).

#### Return Value

If a smart battery is fitted and working correctly and the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of the string parameter will be unchanged.

### psuGetBatteryStatus

#### Description

Retrieves the main rechargeable smart battery status (if one is fitted).

ISISAPI BOOL APIENTRY psuGetBatteryStatus(PISISBATTERYSTATUS pBatteryStatus);

pBatteryStatus	Points to an ISISBATTERYSTATUS structure to hold the status information

#### Remarks

This function reads the current status of the main battery from the micro-controller and returns it at the address passed.

If the status indicates that a battery is fitted, but a fault has occurred the psuGetBatteryInfo() function can be called to obtain further information about the failure.

See chapter 3 for details of the ISISBATTERYSTATUS structure.

#### Return Value

If a smart battery is fitted and working correctly and the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of the string parameter will be unchanged.

### psuGetBatteryInfo

#### Description

Retrieves the main rechargeable smart battery information (if one is fitted).

ISISAPI BOOL APIENTRY psuGetBatteryInfo(PISISBATTERYINFO pBatteryInfo);

pBatteryInfo	Points to an ISISBATTERYINFO structure to hold the information

#### Remarks

This function reads the detailed status information of the main battery from the micro-controller and returns it at the address passed.

See chapter 3 for details of the ISISBATTERYINFO structure.

#### Return Value

If a smart battery is fitted and working correctly and the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of the string parameter will be unchanged.

### psuGetAllowSuspend, psuSetAllowSuspend

#### Description

These functions control whether the unit is permitted to turn off whilst the main battery is being charged.

ISISAPI BOOL APIENTRY psuGetAllowSuspend(PBOOL pbEnabled);

ISISAPI BOOL APIENTRY psuSetAllowSuspend (BOOL bEnabled);

pbEnabled	Points to a BOOL flag for the current status.

bEnabled	BOOL flag indicating the new status.

#### Remarks

Normally the unit is not allowed to power down whilst charging is active since the batteries do not charge when the unit is turned off.  However, by use of the psuSetAllowSuspend function this may be enabled.

This affects all forms of power down, i.e. selecting ‘Suspend’ from the start menu, pressing the power key, or when the system has been configured to time out whilst the A/C adapter is connected (Windows CE user guide documentation of control panel).

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### psuGetChargingEnabled, psuEnableCharging, psuDisableCharging

#### Description

These functions can be used to control whether a fitted main battery can be charged and to detect if charging is currently allowed.

ISISAPI BOOL APIENTRY psuGetChargingEnabled(PBOOL pEnabled);

ISISAPI BOOL APIENTRY psuEnableCharging (void);

ISISAPI BOOL APIENTRY psuDisableCharging (void);

pEnabled	Points to a BOOL flag for the current charging enabled status.

#### Remarks

These functions will only effect the charging status when a rechargeable battery is fitted.  Otherwise the current flag will be read or set and any changes will become effective when a suitable battery is inserted.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### lcdGetBacklight, lcdSetBacklight

#### Description

These functions are used to read and change the backlight level and timeout settings.

ISISAPI BOOL APIENTRY lcdGetBacklight(PISISBACKLIGHT pBackLight);

ISISAPI BOOL APIENTRY lcdSetBacklight (PISISBACKLIGHT pBackLight);

pBackLight	Points to an ISISBACKLIGHT structure

#### Remarks

When using these functions, bits 6 and 7 of Byte 1 and bits 7, 3 and 0 of Byte 2 should always be ignored since they are used by the DLL to flag certain conditions to the micro controller.

When changing the Backlight State, bits 5 and 4 of Byte 2 should also be ignored.

On a colour fex21 the backlight is never truly off.  Consequently the bit 6 of Byte 2 should be ignored, whilst the brightness control bits can be ignored for monochrome units.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### lcdGetContrast, lcdSetContrast

#### Description

These functions are used to read and change the LCD contrast level.

ISISAPI BOOL APIENTRY lcdGetContrast(BYTE* pbGetContrastValue);

ISISAPI BOOL APIENTRY lcdSetContrast (BYTE bGetContrastValue);

pbGetContrastValue	Points to a BYTE to hold the contrast level

bGetContrastValue	Contains the new contrast level

#### Remarks

The contrast can be any value between 0 and 127 where 0 is the lightest and 127 the darkest.

When reading the current value if bit 7 is set, this indicates that the value has been changed by the contrast keys since it was last read.

When changing the value bit 7 should be ignored.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### nvramReadBlock, nvramWriteBlock

#### Description

These functions are used to read and write non-volatile data to the micro controller’s non-volatile E2PROM space.

ISISAPI BOOL APIENTRY nvramReadBlock(DWORD nBlock, BYTE* pData);

ISISAPI BOOL APIENTRY nvramWriteBlock (DWORD nBlock, const BYTE* pData);

nBlock		Contains the number of the 16 Byte block to access

pData		Address of buffer holding new data to write or to receive read data.

#### Remarks

There are 8, 16 Byte blocks of non-volatile E2PROM available for user data which can be accessed by these functions.

The nBlock parameter should be a value from 0 to 7 indicating which block is to be accessed, whilst pData must point to an 8 Byte data area containing either the new data to be written, or available to receive the data from the block.

When data is written to a block, then entire 8 Bytes is written, so the application should be careful to read the block first, change whatever it needs to change and write the updated data back again, in order to avoid data loss in unchanged bytes.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### Example

#define MYDATA_BLOCK     2

#define MYDATA_OFFSET    4

#define MYDATA_SIZE      4

BOOL SaveMyData( PBYTE pMyData )

{

BYTE  bBlock[NV_RAM_BLOCK_SIZE];

PBYTE pDst = & bBlock [MYDATA_OFFSET];

BOOL  bRet = ( pMyData != NULL );

/* Must read the current data first */

if(( bRet )&&( nvramReadBlock( MYDATA_BLOCK, bBlock ))

{

/* Change the Bytes we need to save */

for( UINT uIdx = 0; uIdx < MYDATA_SIZE; uIdx++ )

pDst[uIdx] = pMyData[uIdx];

/* Write the modified data back */

bRet = nvramWriteBlock( MYDATA_BLOCK, bBlock );

}

return bRet;

}

### nvramGetSerialNumber

#### Description

These functions are used to read the unique, fex21, 10 character, unit serial number string.

ISISAPI BOOL APIENTRY nvramGetSerialNumber(TCHAR *szStr);

ISISAPI BOOL APIENTRY nvramGetSerialNumber(TCHAR* szStr);

szStr		Unicode string large enough to hold 10 characters plus a terminating NULL.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### nvramGetUUID, nvramGetUUIDStr

#### Description

These functions are used to read the Universally Unique Identifier.

ISISAPI BOOL APIENTRY nvramGetUUID(UUID* pUUID);

ISISAPI BOOL APIENTRY nvramGetUUIDStr (TCHAR* szStr);

pUUID		Address of a UUID structure (See SDK documentation for details)

szStr		Address of 40 character Unicode string to hold human readable UUID.

#### Remarks

The nvramGetUUID function reads the UUID as a 40bit binary UUID structure as defined in the standard SDK (see below for brief description).

UUIDs uniquely identify objects such as interfaces, manager entry-point vectors, and client objects.  In this case it is used to identify the fex21 unit itself.

#### UUID

typedef struct _GUID {

unsigned long Data1;

unsigned short Data2;

unsigned short Data3;

unsigned char Data4[8];

} GUID;

typedef GUID UUID;

#define uuid_t UUID

| Data1 | Specifies the first eight hexadecimal digits of the UUID. |
| --- | --- |
| Data2 | Specifies the first group of four hexadecimal digits of the UUID. |
| Data3 | Specifies the second group of four hexadecimal digits of the UUID. |
| Data4 | Specifies an array of eight elements. The first two elements contain the third group of four hexadecimal digits of the UUID. The remaining six elements contain the final 12 hexadecimal digits of the UUID. |

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### rtcGetBackup, rtcSetBackup

#### Description

These functions are used access the RTC held in the micro controller, which is used to regulate the charge self discharge features of the battery.

ISISAPI BOOL APIENTRY rtcGetBackup(PISISRTC pRTC);

ISISAPI BOOL APIENTRY rtcSetBackup (PISISRTC pRTC);

pRTC		Address of an ISISRTC structure used to hold time and date information

#### Remarks

It should be unnecessary for a system developer to use these functions, unless they want to synchronise the backup RTC with the system clock and use its value to restore the system clock in the event of a complete power failure or cold reset.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### comGetPort1Connection, comSetPort1Connection

#### Description

These functions are used to change and determine whether the port 1 (COM1:) is directed to use the primary D-Type (main) connector or the Charging and Communications studs.

ISISAPI BOOL APIENTRY comGetPort1Connection (BOOL* pbMainSelected);

ISISAPI BOOL APIENTRY comSetPort1Connection (BOOL bUseMainConnector);

pbMainSelected		Address of a BOOL flag to indicate how the port is connected.

bUseMainConnector	BOOL flag indicting how the port is to be connected.

#### Remarks

A non-zero value for the parameters of these functions indicates that the primary D-Type connector is to be used for COM1: communications.

A zero value for the parameters of these functions indicates that the cradle studs connector is to be used for COM1: communications.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### comGetPort2Connection, comSetPort2Connection

#### Description

These functions are used to change and determine whether the port 2 (COM3: or COM4:) is directed to use the secondary D-Type (main) connector (where fitted) or Infra Red (IR) port.

ISISAPI BOOL APIENTRY comGetPort2Connection (BOOL* pbMainSelected);

ISISAPI BOOL APIENTRY comSetPort2Connection (BOOL bUseMainConnector);

pbMainSelected		Address of a BOOL flag to indicate how the port is connected.

bUseMainConnector	BOOL flag indicting how the port is to be connected.

#### Remarks

A non-zero value for the parameters of these functions indicates that the secondary D-Type connector is to be used for COM3: and COM4: communications.

A zero value for the parameters of these functions indicates that the IR interface is to be used for COM3: and COM4: communications.

The comSetPort2Connection function can only be used whilst the port is not already in use since it accesses the serial.dll driver to change the selection.  A user application may also use this approach by calling the EscapeCommFunction function of the Windows CE API with the SETIR and CLRIR parameters to select the desired connection.

The secondary port is used for both raw IR (COM4:) and Infra red Data Association (IrDA) protocol communications (COM3:).  Using IrDA over the secondary, RS232 D-Type connector is unlikely to be required.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.

### comGetPort1PowerOut, comSetPort1PowerOut, comGetPort2PowerOut, comSetPort2PowerOut

#### Description

These functions are used to change and determine the status of the power output on the primary and secondary D-Type connectors.

ISISAPI BOOL APIENTRY comGetPort1PowerOut (BOOL* pbPowerOut);

ISISAPI BOOL APIENTRY comSetPort1PowerOut (BOOL bPowerOu);

ISISAPI BOOL APIENTRY comGetPort2PowerOut (BOOL* pbPowerOut);

ISISAPI BOOL APIENTRY comSetPort2PowerOut (BOOL bPowerOu);

pbPowerOut	Address of a BOOL flag to indicate If power output is enabled.

bPowerOu	BOOL flag indicting if power output is to be enabled

#### Remarks

Some serial devices (including the optional Integral laser scanner) require power to be supplied by the host computer in order to work.

To save power wherever possible this power output is disabled by default on the fex21, however by use of these functions the application can enable power output on one or both of the D-Type connectors.

It is advisable that the application disables the power once the device is no longer in use to conserve battery life.

A non-zero value for the parameters of these functions indicates that the power output is active.

#### Return Value

If the function call is successful, the function returns TRUE.  Otherwise it returns FALSE and the content of any parameters will remain unchanged.